Workshop:

https://www.youtube.com/watch?v=NXECj8eloYY

https://www.youtube.com/watch?v=TIuLOQ-jXko

https://www.youtube.com/watch?v=5EqyURlZDMA

Preprocess EEG DATA:

- Install matlab toolboxes (EEGLab, FMRIB eeglab plugin)

  - https://sccn.ucsd.edu/eeglab/download.php

(Run eeglab command and File -> Manage EEGLab Extensions ; look for the plugin)

  - https://fsl.fmrib.ox.ac.uk/eeglab/fmribplugin/#whatis

  - To remove line noise: Download the CleanLine plugin and place it in your EEGLAB plugins folder

- EEG LAB:

(Every step of preprocessing asks for the data storage option; it is better to override existing rather than storing separate files)

- Run eeglab

- import data .vhdr files

- plot and see how it looks

- Remove fmri gradients  (requires high sampling rate)

  - For non-eeg channel no. info; open the EEG object loaded and check inside EEG.chanlogs variable for channel names and manually determine non-eeg channel numbers (eg: ECG, EOG*). On our case, it is just ECG value 32.

  - open Tools->fMRIB -> FastR remove gradients (LPF:0 ; Interpolation folds:1; window length : 10; artifact event slice : x 1; non-eeg channel no.: 32)

  - plot and check how the data looks

- Detect QRS events (FMRIB Tools -> Detect QRS Events)

  - ECG channel no: 32 (as determined in step 5a); “qrs”; delete..: no

  - This plugin is not maintained properly. So if it fails, remove a few initial TR’s and then check..

  - Once detected, reduced sampling..and the remove

- Remove pulse artifact:

  - Event: “qrs” ;

  - Select optimal basis set; No. of PCs: 3 (can be changed based on dataset)

- Change sampling rate from default to “1000”

- Filter data (Basic FIR Filter). --- can be done anywhere (prefer doing before fmrib, as it helps … ) Tools -> Filter Data -> Basic FIR Filter

  - It is recommended not to do band pass filtering for EEG data. Do low pass first and then high pass

  - Low pass value: 1 and click ok

  - High pass filter value: 30 (used this; need to check what would fit our data)

- Plot and check the data

- Re-reference data (select avg) (Tools-> re-referen…) [QUES: EEG data is recorded based on a reference point as they are voltages, this step tries to re-reference the data.. Need to understand more what exactly this does) needs to …..

- Perform ICA decomposition (Tools->Decompose Data by ICA)

  - Extended infomax algorithm; Select “Reroder components by variance); click “OK”

  - This step takes time

- Save the eeg data (EEG variable as a mat file) as the next labelling step cannot be done from CLI.  (SCRIPT AVAILABLE TILL HERE)

- Manually load the mat file (import->mat-> update sampling to 1000)

  - File-> Importdata->Using EEGLab…-> From Matlab array

- After loading ICA output, click Tools -> Classify Components by ICLabel -> Label Components; then (select default)

  - QUES: Why can’t we do this and then save?

- It shows all the components. Choose the ones with high brain percentage. May be >85% (Threshold based on experience)

- Filter components (Tools ->Remove Comp from data)

  - DO NOT OVERWRITE

- Remove ECG channel (edit -> select data), at this step that the data can be overwritten during saving. (QUES: Is this step already done in the script before doing ICA?)

- Save the corresponding changed data in EEG object. (Saved as .mat format)

- Save current dataset (File -> Save current dataset). This is saved as “.set” and “.fdt” files.

- Data segmentation to consider data which has corresponding fMRI data recorded. (QUES: Do we do this for each eeg file corresponding to task for a subject? Does this need to same for a task across all the subjects? Not in the script.. Does it need to be done before or after ICA? Online suggests after ICA.)

  - Before Analysis: Position of this step -- confirm with Souvik...

  - Notes: [Trigger + TR]

  - After the MRI scanner sends the trigger “X1” in our case, there is a delay between the trigger time and the actual MRI data recording time..usally [trigger point + TR], this is the point where the EEG and MRI data are actually being recorded in-parallel

- This concludes the EEG preprocessing.

POINTS TO CONSIDER:

- Sometimes there is missing data or null’s found, what needs to be done

  - Souvik suggested interpolate the value based on the neighboring average

- Missing “x1” trigger

- Data segmentation guidelines

EEGLAB Notes:

- ALLCOM is a stack variable stores all the commands that are executed while we perform GUI tasks..(since it is LIFO order, we need to print in reverse order to get the commands in the order we have executed in the GUI). Use the following command to print the commands in the order we have executed in the GUI

  - celldisp(flip(ALLCOM))

- ALLEEG variable has all the instances of the EEG data for all the preprocessing steps performed for an eeg data

- Whenever we overwrite the data, it is always saving in the EEG variable and not in a file.

19-Oct-2025: Commands in matlab for sub01  - RS1 : Repeating exp steps done in 17-Oct-2025:

Commands:

load('sub1_rs1_eeg_preprocessed.mat', 'EEG')

eeglab redraw;

[ALLEEG EEG CURRENTSET ALLCOM] = eeglab;

[EEG ALLEEG CURRENTSET] = eeg_retrieve(ALLEEG,1);

EEG = pop_iclabel(EEG, 'default')

[ALLEEG, EEG, CURRENTSET] = eeg_store(ALLEEG, EEG, CURRENTSET);

EEG = pop_subcomp( EEG, [3   4   5   6   8   9  11  12  16  17  19  21  27  28  30  32  36  39  43  44  46  47  48  49  52  62], 0);

[ALLEEG EEG CURRENTSET] = pop_newset(ALLEEG, EEG, 1,'gui','off');

EEG = pop_select( EEG, 'rmchannel',{'ECG'});

[ALLEEG EEG CURRENTSET] = pop_newset(ALLEEG, EEG, 2,'overwrite','on','gui','off');

#Saved EEG as .mat file

EEG = pop_saveset( EEG, 'filename','test.set','filepath','/Users/sbasodi1/GSU Dropbox Dropbox/Sunitha Basodi (sbasodi1)/snt_mac/workplace/sym_cabi/SYM_CABI_MeditationStudy/DataPreprocessing/');

[ALLEEG, EEG, CURRENTSET] = eeg_store(ALLEEG, EEG, CURRENTSET);

17-Oct-2025: Commands in matlab for sub01  - RS1 with Souvik:

pop_eegplot( EEG, 1, 1, 1);

[ALLEEG EEG CURRENTSET] = pop_newset(ALLEEG, EEG, 2,overwrite,on,gui,off);

EEG = pop_select( EEG, rmchannel,{ECG});

pop_eegplot( EEG, 1, 1, 1);

[ALLEEG EEG CURRENTSET] = pop_newset(ALLEEG, EEG, 1,savenew,sub1_rs1_eeg_ica_filtered,gui,off);

EEG = pop_subcomp( EEG, [3   4   5   6   8   9  11   12  16  17  19  21  27  28  30  32  36  39  43  44  46  47  48  49  52  62], 0);

[ALLEEG, EEG, CURRENTSET] = eeg_store(ALLEEG, EEG, CURRENTSET);

EEG = pop_iclabel(EEG, default);

pop_eegplot( EEG, 1, 1, 1);

eeglab(redraw);

[ALLEEG EEG CURRENTSET] = pop_newset(ALLEEG, EEG, 0,setname,eee,gui,off);

EEG = pop_importdata(dataformat,matlab,nbchan,0,data,/Users/sbasodi1/GSU Dropbox Dropbox/Sunitha Basodi (sbasodi1)/snt_mac/workplace/sym_cabi/SYM_CABI_MeditationStudy/DataPreprocessing/sub1_rs1_eeg_preprocessed.mat,srate,1000,pnts,0,xmin,0);

[ALLEEG EEG CURRENTSET ALLCOM] = eeglab;

Notes:

SOM HRF — To handle delay

Currently, remove first three volumes of fMRI

Retain: 3:6,8,9,11,1,2,16,17,9,21,27,28,30,32,36,39,43,44,46:49,52,62

Running all preprocessing steps in GUI dec-18

[ALLEEG EEG CURRENTSET ALLCOM] = eeglab;

EEG = pop_loadbv('/Users/sbasodi1/GSU Dropbox Dropbox/Sunitha Basodi (sbasodi1)/snt_mac/workplace/sym_cabi/SYM_CABI_MeditationStudy/DataPreprocessing/data/raw_all/SUB01/SUB_001_12182024_EEG/', 'SYM_CABI_RSfMRI_1_SUB01.vhdr', [1 2719000], [1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 31 32 33 34 35 36 37 38 39 40 41 42 43 44 45 46 47 48 49 50 51 52 53 54 55 56 57 58 59 60 61 62 63 64]);

[ALLEEG EEG CURRENTSET] = pop_newset(ALLEEG, EEG, 0,'setname','aaa','gui','off');

EEG = pop_fmrib_fastr(EEG,0,10,30,'X  1',1,0,0,0,0,0.03,[32],'auto');

[ALLEEG EEG CURRENTSET] = pop_newset(ALLEEG, EEG, 1,'overwrite','on','gui','off');

pop_eegplot( EEG, 1, 1, 1);

EEG=pop_fmrib_qrsdetect(EEG,32,'qrs','no')

[ALLEEG EEG CURRENTSET] = pop_newset(ALLEEG, EEG, 1,'overwrite','on','gui','off');

EEG = pop_resample( EEG, 1000);

[ALLEEG EEG CURRENTSET] = pop_newset(ALLEEG, EEG, 1,'overwrite','on','gui','off');

EEG = pop_fmrib_pas(EEG,'qrs','obs',3);

[ALLEEG EEG CURRENTSET] = pop_newset(ALLEEG, EEG, 1,'overwrite','on','gui','off');

pop_eegplot( EEG, 1, 1, 1);

EEG = pop_eegfiltnew(EEG, 'locutoff',1,'plotfreqz',1);

[ALLEEG EEG CURRENTSET] = pop_newset(ALLEEG, EEG, 1,'overwrite','on','gui','off');

EEG = pop_eegfiltnew(EEG, 'hicutoff',30,'plotfreqz',1);

[ALLEEG EEG CURRENTSET] = pop_newset(ALLEEG, EEG, 1,'overwrite','on','gui','off');

pop_eegplot( EEG, 1, 1, 1);

EEG = pop_reref( EEG, []);

[ALLEEG EEG CURRENTSET] = pop_newset(ALLEEG, EEG, 1,'overwrite','on','gui','off');

EEG = pop_cleanline(EEG, 'bandwidth',2,'chanlist',[1:64] ,'computepower',1,'linefreqs',60,'newversion',0,'normSpectrum',0,'p',0.01,'pad',2,'plotfigures',0,'scanforlines',0,'sigtype','Channels','taperbandwidth',2,'tau',100,'verb',1,'winsize',4,'winstep',1);

[ALLEEG EEG CURRENTSET] = pop_newset(ALLEEG, EEG, 1,'overwrite','on','gui','off');

EEG = pop_runica(EEG, 'icatype', 'runica', 'extended',1,'rndreset','yes','interrupt','on','pca',63);

[ALLEEG, EEG, CURRENTSET] = eeg_store(ALLEEG, EEG, CURRENTSET);